Restore Calcium & Lime Remover

Issue Date 18-Aug-2015	Revision Date 24-Feb-2015

SAFETY DATA SHEET

## Version 1

1. PRODUCT AND COMPANY IDENTIFICATION

Product identifier

Product Name	 Restore Calcium & Lime Remover

## Other means of identification

Product Code	CB-290

Synonyms	None

## Details of the supplier of the safety data sheet

Company Name	P AND C SUPPLIES

35 Tonique darling Hwy

242-364-3505

## Emergency telephone number

Emergency Telephone	Chemtrec 1-242-427-4923

2. HAZARDS IDENTIFICATION

## Classification

OSHA Regulatory Status

This chemical is considered hazardous by the 2012 OSHA Hazard Communication Standard (29 CFR 1910.1200)

| Acute toxicity - Oral | Not classified |
| --- | --- |
| Acute toxicity - Dermal | Not classified |
| Skin corrosion/irritation | Category 3 |

## Label elements

Warning

## Emergency Overview

Hazard statements

Causes mild skin irritation

Appearance Clear	Physical state Liquid	Odor Fresh & Clean

## Precautionary Statements - Response

Immediately call a POISON CENTER or doctor/physician Specific Treatment (See Section 4 on the SDS)

If skin irritation occurs: Get medical advice/attention

## Hazards not otherwise classified (HNOC) Other Information

Unknown Acute Toxicity	0% of the mixture consists of ingredient(s) of unknown toxicity

3. COMPOSITION/INFORMATION ON INGREDIENTS

# Page  1 / 6

| Chemical Name | CAS No. | Weight-% | Trade Secret |
| --- | --- | --- | --- |
| 2-Hydroxypropanoic acid | 79-33-4 | 10-30 | * |
| PROPRIETARY | Proprietary | 1-5 | * |

*The exact percentage (concentration) of composition has been withheld as a trade secret.

4. FIRST AID MEASURES

## First aid measures

Skin Contact	Wash off immediately with plenty of water. Wash skin with soap and water.

Eye contact	Rinse thoroughly with plenty of water for at least 15 minutes, lifting lower and upper eyelids.

Consult a physician.

Inhalation	Remove to fresh air.

Ingestion	Clean mouth with water and drink afterwards plenty of water.

## Most important symptoms and effects, both acute and delayed

Symptoms	Any additional important symptoms and effects are described in Section 11: Toxicology Information.

## Indication of any immediate medical attention and special treatment needed Note to physicians	Treat symptomatically.

5. FIRE-FIGHTING MEASURES

Suitable extinguishing media

Use extinguishing measures that are appropriate to local circumstances and the surrounding environment.

Unsuitable extinguishing media Caution: Use of water spray when fighting fire may be inefficient.

## Specific hazards arising from the chemical

No Information available.

## Explosion data

Sensitivity to Mechanical Impact None.

Sensitivity to Static Discharge	None.

Protective equipment and precautions for firefighters

As in any fire, wear self-contained breathing apparatus pressure-demand, MSHA/NIOSH (approved or equivalent) and full protective gear.

6. ACCIDENTAL RELEASE MEASURES

## Personal precautions, protective equipment and emergency procedures

Personal precautions	Ensure adequate ventilation, especially in confined areas.

## Environmental precautions

Environmental precautions	See Section 12 for additional ecological information.

## Methods and material for containment and cleaning up

Methods for containment	Prevent further leakage or spillage if safe to do so.

Methods for cleaning up	Pick up and transfer to properly labeled containers.

7. HANDLING AND STORAGE

## Precautions for safe handling

Advice on safe handling	Handle in accordance with good industrial hygiene and safety practice.

## Conditions for safe storage, including any incompatibilities

Storage Conditions	Keep containers tightly closed in a dry, cool and well-ventilated place.

Incompatible materials	None known based on information supplied.

8. EXPOSURE CONTROLS/PERSONAL PROTECTION

## Control parameters

Exposure Guidelines	This product, as supplied, does not contain any hazardous materials with occupational

exposure limits established by the region specific regulatory bodies.

## Appropriate engineering controls

Engineering Controls	Showers, Eyewash stations & Ventilation systems.

## Individual protection measures, such as personal protective equipment

Eye/face protection	Wear safety glasses with side shields (or goggles).

Skin and body protection	Rubber gloves if prolonged contact and/or handling large volumes.

Respiratory protection	Ensure adequate ventilation, especially in confined areas. If exposure limits are exceeded or irritation is experienced, NIOSH/MSHA approved respiratory protection should be worn. Positive-pressure supplied air respirators or air purifying respirators may be required for high airborne contaminant concentrations. Respiratory protection must be provided in accordance with current local regulations.

General Hygiene	Handle in accordance with good industrial hygiene and safety practice.

9. PHYSICAL AND CHEMICAL PROPERTIES

# Information on basic physical and chemical properties

Physical state	Liquid

# Appearance

Clear

Color	Colorless

# Odor

Fresh & Clean

Odor threshold	No Information available

# Property

## pH

Specific Gravity Viscosity

Values

2.0 - 2.5

1.03

< 25 cP @ 25°C

# Remarks • Method

Melting point/freezing point	No Information available

Flash point	Above 200°F

Boiling point / boiling range	> 212 ° F

Evaporation rate	No Information available Flammability (solid, gas)	No data available Flammability Limits in Air

Upper flammability limit:	No Information available Lower flammability limit:	No Information available Vapor pressure	No Information available

Vapor density	No Information available

Water solubility	Complete

Partition coefficient	No Information available Autoignition temperature	No Information available Decomposition temperature	No Information available

# Other Information

## Density Lbs/Gal	8.56

10. STABILITY AND REACTIVITY

VOC Content (%)	Not Applicable

## Reactivity

No data available

## Chemical stability

Stable under recommended storage conditions.

## Possibility of Hazardous Reactions

None under normal processing.

## Conditions to avoid

Extremes of temperature and direct sunlight.

## Incompatible materials

None known based on information supplied. Hazardous Decomposition Products None known based on information supplied.

11. TOXICOLOGICAL INFORMATION

## Information on likely routes of exposure

Product Information	No acute toxicity information is available for this product

Inhalation	Inhalation of vapors in high concentration may cause irritation of respiratory system.

Breathing of vapors may cause dizziness and shortness of breath.

Eye contact	Avoid contact with eyes. Contact with eyes may cause irritation.

Skin Contact	Avoid contact with skin and clothing. Causes skin irritation.

Ingestion	May cause stomach distress, nausea, or vomiting. Do not taste or swallow.

| Chemical Name | Oral LD50 | Dermal LD50 | Inhalation LC50 |
| --- | --- | --- | --- |
| 2-Hydroxypropanoic acid 79-33-4 | = 3730 mg/kg ( Rat ) | > 2000 mg/kg ( Rabbit ) | - |
| PROPRIETARY | 2546 mg/kg ( Rat ) | 1844 mg/kg ( Rat ) | - |

## Information on toxicological effects

Symptoms	No Information available.

## Delayed and immediate effects as well as chronic effects from short and long-term exposure Sensitization	No Information available.

Germ cell mutagenicity	No Information available.

Carcinogenicity	No Information available.

Reproductive toxicity	No Information available.

STOT - single exposure	No Information available. STOT - repeated exposure	No Information available. Aspiration hazard	No Information available.

## Numerical measures of toxicity - Product Information

Unknown Acute Toxicity	0% of the mixture consists of ingredient(s) of unknown toxicity

## The following values are calculated based on chapter 3.1 of the GHS document .

ATEmix (oral)	23,124.00

ATEmix (dermal)	13,024.00

12. ECOLOGICAL INFORMATION

## Ecotoxicity

0.5% of the mixture consists of components(s) of unknown hazards to the aquatic environment

| Chemical Name | Algae/aquatic plants | Fish | Crustacea |
| --- | --- | --- | --- |
| 2-Hydroxypropanoic acid 79-33-4 | 3.5: 70 h Pseudokirchneriella subcapitata mg/L EC50 | 320: 96 h Brachydanio rerio mg/L LC50 semi-static 100 - 180: 96 h Lepomis macrochirus mg/L LC50 static 100 - 180: 96 h Oncorhynchus mykiss mg/L LC50 static | 240: 48 h Daphnia magna mg/L EC50 180 - 320: 48 h Daphnia magna mg/L EC50 Static |
| PROPRIETARY | - | 32.15: 96 h Rainbow Trout mg/L LC50 | - |

## Persistence and degradability

No Information available.

## Bioaccumulation

Bioaccumulative potential.

| Chemical Name | Partition coefficient |
| --- | --- |
| 2-Hydroxypropanoic acid 79-33-4 | -0.62 |

Other adverse effects	No Information available

13. DISPOSAL CONSIDERATIONS

## Waste treatment methods

Disposal of wastes	Disposal should be in accordance with applicable regional, national and local laws and

regulations.

Contaminated packaging	Do not reuse container.

14. TRANSPORT INFORMATION

The basic description below is specific to the container size. This information is provided for at a glance DOT information. Please refer to the container and/or shipping papers for the appropriate shipping description before tendering this material for shipment. For additional information, please contact the distributor listed in section 1 of this SDS.

DOT	Not regulated

TDG	Not regulated

15. REGULATORY INFORMATION

# International Inventories

TSCA	Complies

DSL/NDSL	Complies

# Legend:

TSCA - United States Toxic Substances Control Act Section 8(b) Inventory

DSL/NDSL - Canadian Domestic Substances List/Non-Domestic Substances List

# US Federal Regulations

## SARA 313

Section 313 of Title III of the Superfund Amendments and Reauthorization Act of 1986 (SARA). This product does not contain any

chemicals which are subject to the reporting requirements of the Act and Title 40 of the Code of Federal Regulations, Part 372

## SARA 311/312 Hazard Categories

Acute health hazard	No

Chronic Health Hazard	No

Fire hazard	No

Sudden release of pressure hazard	No

Reactive Hazard	No

CWA (Clean Water Act)

This product does not contain any substances regulated as pollutants pursuant to the Clean Water Act (40 CFR 122.21 and 40 CFR 122.42)

## CERCLA

This material, as supplied, does not contain any substances regulated as hazardous substances under the Comprehensive Environmental Response Compensation and Liability Act (CERCLA) (40 CFR 302) or the Superfund Amendments and Reauthorization Act (SARA) (40 CFR 355). There may be specific reporting requirements at the local, regional, or state level pertaining to releases of this material

# US State Regulations

## California Proposition 65

This product does not contain any Proposition 65 chemicals

## U.S. State Right-to-Know Regulations

U.S. EPA Label Information

EPA Pesticide Registration Number Not Applicable

16. OTHER INFORMATION

## NFPA

Health hazards 0	Flammability 0	Instability 0	Physical and Chemical

Properties Yes

## HMIS

Health hazards 0	Flammability 0

Physical hazards 0

Personal protection B

Issue Date	18-Aug-2015

Revision Date	24-Feb-2015

## Revision Note

No Information available

## Disclaimer

The information provided in this Safety Data Sheet is correct to the best of our knowledge, information and belief at the date of its publication. The information given is designed only as a guidance for safe handling, use, processing, storage, transportation, disposal and release and is not to be considered a warranty or quality specification. The information relates only to the specific material designated and may not be valid for such material used in combination with any other materials or in any process, unless specified in the text.

End of Safety Data Sheet