SAFETY DATA SHEET

P AND C SUPPLIES

SURE GRIP

Issue Date 21-Mar-2018	Revision Date 21-Mar-2018	Version 1

1. PRODUCT AND COMPANY IDENTIFICATION

## Product identifier

Product Name	Bulk

## Other means of identification

Product Code	F3910

Synonyms	None

## Details of the supplier of the safety data sheet

Company Name                               P AND C SUPPLIES

35 Tonique Darling Hwy

Nassau, Bahamas

242-364-3505

## Emergency telephone number

Emergency Telephone	 1-242-427-4923

2. HAZARDS IDENTIFICATION

## Classification

OSHA Regulatory Status

This chemical is considered hazardous by the 2012 OSHA Hazard Communication Standard (29 CFR 1910.1200)

| Acute toxicity - Oral | Not classified |
| --- | --- |
| Acute toxicity - Dermal | Not classified |
| Skin corrosion/irritation | Category 1 |
| Serious eye damage/eye irritation | Category 1 |

## Label elements

Emergency Overview

Danger

## Hazard statements

Causes severe skin burns and eye damage

Appearance Clear Green	Physical state Liquid	Odor Citrus

Page  1 / 8

## Precautionary Statements - Prevention

Do not breathe dust/fume/gas/mist/vapors/spray

Wash face, hands and any exposed skin thoroughly after handling Wear protective gloves/protective clothing/eye protection/face protection

## Precautionary Statements - Response

Specific Treatment (See Section 4 on the SDS)

IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing Immediately call a POISON CENTER or doctor/physician

IF ON SKIN (or hair): Remove/Take off immediately all contaminated clothing. Rinse skin with water/shower Wash contaminated clothing before reuse

IF INHALED: Remove victim to fresh air and keep at rest in a position comfortable for breathing Immediately call a POISON CENTER or doctor/physician

IF SWALLOWED: Rinse mouth. DO NOT induce vomiting Drink plenty of water

Immediately call a POISON CENTER or doctor/physician

## Precautionary Statements - Storage

Store locked up

## Precautionary Statements - Disposal

Disposal should be in accordance with applicable regional, national and local laws and regulations

## Hazards not otherwise classified (HNOC) Other Information

- Harmful to aquatic life with long lasting effects

- Harmful to aquatic life

Unknown Acute Toxicity	0% of the mixture consists of ingredient(s) of unknown toxicity

3. COMPOSITION/INFORMATION ON INGREDIENTS

| Chemical Name | CAS No. | Weight-% | Trade Secret |
| --- | --- | --- | --- |
| Sodium Dodecylbenzene Sulfonate | 25155-30-0 | 1-5 | * |
| Glycolic Acid | 79-14-1 | 1-5 | * |
| Nonylphenol Ethoxylate | 9016-45-9 | 1-5 | * |
| Citric Acid | 77-92-9 | 1-5 | * |

*The exact percentage (concentration) of composition has been withheld as a trade secret.

4. FIRST AID MEASURES

## First aid measures

General advice	Immediate medical attention is required.

Skin Contact	Immediate medical attention is required. Wash off immediately with soap and plenty of water while removing all contaminated clothes and shoes.

Eye contact	Immediate medical attention is required. Rinse immediately with plenty of water, also under the eyelids, for at least 15 minutes. Keep eye wide open while rinsing. Do not rub affected area.

Inhalation	Remove to fresh air. Call a physician or poison control center immediately. If not breathing, give artificial respiration. If breathing is difficult, give oxygen.

Ingestion	Immediate medical attention is required. Do NOT induce vomiting. Drink plenty of water.

Never give anything by mouth to an unconscious person. Remove from exposure, lie down. Clean mouth with water and drink afterwards plenty of water. Call a physician or poison control center immediately.

Self-protection of the first aider	Use personal protective equipment as required. Avoid contact with skin, eyes or clothing.

## Most important symptoms and effects, both acute and delayed

Symptoms	Any additional important symptoms and effects are described in Section 11: Toxicology Information.

## Indication of any immediate medical attention and special treatment needed

Note to physicians	Product is a corrosive material. Use of gastric lavage or emesis is contraindicated.

Possible perforation of stomach or esophagus should be investigated. Do not give chemical antidotes. Asphyxia from glottal edema may occur. Marked decrease in blood pressure may occur with moist rales, frothy sputum, and high pulse pressure. Treat symptomatically.

5. FIRE-FIGHTING MEASURES

## Suitable extinguishing media

Use extinguishing measures that are appropriate to local circumstances and the surrounding environment.

Unsuitable extinguishing media Caution: Use of water spray when fighting fire may be inefficient.

## Specific hazards arising from the chemical

The product causes burns of eyes, skin and mucous membranes. Thermal decomposition can lead to release of irritating and toxic gases and vapors. In the event of fire and/or explosion do not breathe fumes.

## Explosion data

Sensitivity to Mechanical Impact None.

Sensitivity to Static Discharge	None.

Protective equipment and precautions for firefighters

As in any fire, wear self-contained breathing apparatus pressure-demand, MSHA/NIOSH (approved or equivalent) and full protective gear.

6. ACCIDENTAL RELEASE MEASURES

## Personal precautions, protective equipment and emergency procedures

Personal precautions	Evacuate personnel to safe areas. Use personal protective equipment as required. Avoid

contact with skin, eyes or clothing. Keep people away from and upwind of spill/leak.

## Environmental precautions

Environmental precautions	Do not allow into any storm sewer drains, lakes, streams, ponds, estuaries, oceans or other

surface water bodies. Should not be released into the environment. Dispose of according to all local city, state and federal rules and regulations.

## Methods and material for containment and cleaning up

Methods for containment	Prevent further leakage or spillage if safe to do so.

Methods for cleaning up	Dike far ahead of liquid spill for later disposal. Soak up with inert absorbent material. Take

up mechanically, placing in appropriate containers for disposal. Clean contaminated surface thoroughly. Prevent product from entering drains. Dam up. After cleaning, flush away traces with water.

7. HANDLING AND STORAGE

## Precautions for safe handling

Advice on safe handling	Use personal protective equipment as required. Avoid contact with skin, eyes or clothing.

Ensure adequate ventilation, especially in confined areas. In case of insufficient ventilation, wear suitable respiratory equipment. Use only with adequate ventilation and in closed systems.

## Conditions for safe storage, including any incompatibilities

Storage Conditions	Keep container tightly closed in a dry and well-ventilated place. Keep out of the reach of

children. Keep containers tightly closed in a dry, cool and well-ventilated place. Keep in properly labeled containers.

Incompatible materials	Incompatible with strong acids and bases. Incompatible with oxidizing agents.

8. EXPOSURE CONTROLS/PERSONAL PROTECTION

## Control parameters

Appropriate engineering controls

Engineering Controls	Showers, Eyewash stations & Ventilation systems.

## Individual protection measures, such as personal protective equipment

Eye/face protection	Tight sealing safety goggles. Wear a face shield if splashing hazard exists.

Skin and body protection	Wear impervious protective clothing, including boots, gloves, lab coat, apron or coveralls, as appropriate, to prevent skin contact.

Respiratory protection	If exposure limits are exceeded or irritation is experienced, NIOSH/MSHA approved respiratory protection should be worn. Positive-pressure supplied air respirators may be required for high airborne contaminant concentrations. Respiratory protection must be provided in accordance with current local regulations.

General Hygiene	When using do not eat, drink or smoke. Keep away from food, drink and animal feeding

stuffs. Contaminated work clothing should not be allowed out of the workplace. Regular cleaning of equipment, work area and clothing is recommended. Avoid contact with skin, eyes or clothing. Take off all contaminated clothing and wash it before reuse. Wear suitable gloves and eye/face protection.

9. PHYSICAL AND CHEMICAL PROPERTIES

# Information on basic physical and chemical properties

Physical state	Liquid

Appearance	Clear Green

Color	Green

Odor	Citrus

Odor threshold	No Information available

# Property	Values	Remarks • Method

pH	1.5 - 2.0

## Specific Gravity	1.04

Viscosity	Slightly Viscous

Melting point/freezing point No Information available

Flash point	None

Boiling point / boiling range	100 °C / 212 ° F Degrees Evaporation rate	No Information available Flammability (solid, gas)	No data available Flammability Limits in Air

Upper flammability limit:	No Information available Lower flammability limit:	No Information available Vapor pressure	No Information available

Vapor density	No Information available

Water solubility	Complete

Partition coefficient	No Information available Autoignition temperature	No Information available Decomposition temperature	No Information available

# Other Information

## Density Lbs/Gal	8.66

10. STABILITY AND REACTIVITY

VOC Content (%)	Not Applicable

## Reactivity

No data available

## Chemical stability

Stable under recommended storage conditions.

## Possibility of Hazardous Reactions

None under normal processing.

## Conditions to avoid

Exposure to air or moisture over prolonged periods.

## Incompatible materials

Incompatible with strong acids and bases. Incompatible with oxidizing agents.

## Hazardous Decomposition Products

Thermal decomposition can lead to release of irritating and toxic gases and vapors.

11. TOXICOLOGICAL INFORMATION

## Information on likely routes of exposure

Product Information	The primary effects and toxicity of this material are due to it corrosive nature.

Inhalation	Avoid breathing vapors or mists. Breathing of vapor can cause respiratory irritation and inflammation. Breathing of mist or liquid can cause burns to the respiratory tract.

Eye contact	Avoid contact with eyes. Corrosive. Causes severe eye damage.

Skin Contact	Avoid contact with skin. Corrosive. Contact with skin may cause severe irritation and burns.

Ingestion	Do not taste or swallow. Ingestion causes acute irritation and burns to the mucous membranes of the mouth, trachea, esophagus and stomach.

| Chemical Name | Oral LD50 | Dermal LD50 | Inhalation LC50 |
| --- | --- | --- | --- |
| Sodium Dodecylbenzene Sulfonate 25155-30-0 | = 438 mg/kg ( Rat ) = 500 mg/kg ( Rat ) = 2050 mg/kg ( Rat ) | - | - |
| Glycolic Acid 79-14-1 | = 1950 mg/kg ( Rat ) | - | = 3.6 mg/L ( Rat ) 4 h |
| Nonylphenol Ethoxylate 9016-45-9 | = 2590 mg/kg ( Rat ) = 1310 mg/kg ( Rat ) | = 1780 µL/kg ( Rabbit ) = 2 mL/kg ( Rabbit ) | - |
| Citric Acid 77-92-9 | = 3 g/kg ( Rat ) = 3000 mg/kg ( Rat ) | - | - |

## Information on toxicological effects

Symptoms	No Information available.

## Delayed and immediate effects as well as chronic effects from short and long-term exposure

Corrosivity	Causes burns. Extremely corrosive and destructive to tissue. Risk of serious damage to eyes.

Sensitization	No Information available.

Germ cell mutagenicity	No Information available.

Carcinogenicity	No Information available.

Reproductive toxicity	No Information available.

STOT - single exposure	No Information available.

STOT - repeated exposure	No Information available.

Chronic toxicity	Chronic exposure to corrosive fumes/gases may cause erosion of the teeth followed by jaw necrosis. Bronchial irritation with chronic cough and frequent attacks of pneumonia are common. Gastrointestinal disturbances may also be seen. Avoid repeated exposure.

Possible risk of irreversible effects.

Aspiration hazard	No Information available.

## Numerical measures of toxicity - Product Information

Unknown Acute Toxicity	0% of the mixture consists of ingredient(s) of unknown toxicity

## The following values are calculated based on chapter 3.1 of the GHS document .

| ATEmix (oral) | 7,080.00 |
| --- | --- |
| ATEmix (dermal) | 50,704.57 |
| ATEmix (inhalation-dust/mist) | 82.30 |
|  | 12. ECOLOGICAL INFORMATION |
| Ecotoxicity |  |

0.75% of the mixture consists of components(s) of unknown hazards to the aquatic environment

| Chemical Name | Algae/aquatic plants | Fish | Crustacea |
| --- | --- | --- | --- |
| Sodium Dodecylbenzene Sulfonate 25155-30-0 | - | 10.8: 96 h Oncorhynchus mykiss mg/L LC50 static | - |
| Glycolic Acid 79-14-1 | - | 5000: 96 h Brachydanio rerio mg/L LC50 static | - |
| Nonylphenol Ethoxylate 9016-45-9 | - | 5: 96 h Fish mg/L LC50 | - |
| Citric Acid 77-92-9 | - | 1516: 96 h Lepomis macrochirus mg/L LC50 static | 120: 72 h Daphnia magna mg/L EC50 |
| Sodium Sulfate 7757-82-6 | - | 3040 - 4380: 96 h Lepomis macrochirus mg/L LC50 static 13500: 96 h Lepomis macrochirus mg/L LC50 13500 - 14500: 96 h Pimephales promelas mg/L LC50 6800: 96 h Pimephales promelas mg/L LC50 static | 2564: 48 h Daphnia magna mg/L EC50 630: 96 h Daphnia magna mg/L EC50 |

## Persistence and degradability

No Information available.

## Bioaccumulation

Bioaccumulative potential.

| Chemical Name | Partition coefficient |
| --- | --- |
| Glycolic Acid 79-14-1 | -1.11 |
| Citric Acid 77-92-9 | -1.72 |

Other adverse effects	No Information available

13. DISPOSAL CONSIDERATIONS

## Waste treatment methods

Disposal of wastes	Disposal should be in accordance with applicable regional, national and local laws and

regulations.

Contaminated packaging	Do not reuse container.

14. TRANSPORT INFORMATION

The basic description below is specific to the container size. This information is provided for at a glance DOT information. Please refer to the container and/or shipping papers for the appropriate shipping description before tendering this material for shipment. For additional information, please contact the distributor listed in section 1 of this SDS.

## DOT

UN/ID No.	UN1760

Proper shipping name	Corrosive liquids, n.o.s.

## Hazard Class	8

Packing Group	III

Special Provisions	IB3, T7, TP1, TP28

Description	UN1760, Corrosive liquids, n.o.s. (contains Hydroxyacetic Acid and Dodecylbenzenesulfonic Acid), 8, III

## Emergency Response Guide Number

154

## TDG

UN/ID No.	UN1760

Proper shipping name	Corrosive liquids, n.o.s.

## Hazard Class	8

Packing Group	III

Description	UN1760, Corrosive liquids, n.o.s. (contains Hydroxyacetic Acid and Dodecylbenzenesulfonic Acid), 8, III

15. REGULATORY INFORMATION

# International Inventories

TSCA	Complies

DSL/NDSL	Complies

# Legend:

TSCA - United States Toxic Substances Control Act Section 8(b) Inventory

DSL/NDSL - Canadian Domestic Substances List/Non-Domestic Substances List

# US Federal Regulations

## SARA 313

Section 313 of Title III of the Superfund Amendments and Reauthorization Act of 1986 (SARA). This product does not contain any chemicals which are subject to the reporting requirements of the Act and Title 40 of the Code of Federal Regulations, Part 372

## SARA 311/312 Hazard Categories

Acute health hazard	Yes

Chronic Health Hazard	Yes

Fire hazard	No

Sudden release of pressure hazard	No

Reactive Hazard	No

CWA (Clean Water Act)

This product contains the following substances which are regulated pollutants pursuant to the Clean Water Act (40 CFR 122.21 and 40 CFR 122.42)

| Chemical Name | CWA - Reportable Quantities | CWA - Toxic Pollutants | CWA - Priority Pollutants | CWA - Hazardous Substances |
| --- | --- | --- | --- | --- |
| Sodium Dodecylbenzene Sulfonate 25155-30-0 | 1000 lb | - | - | X |

## CERCLA

This material, as supplied, contains one or more substances regulated as a hazardous substance under the Comprehensive Environmental Response Compensation and Liability Act (CERCLA) (40 CFR 302)

| Chemical Name | Hazardous Substances RQs | CERCLA/SARA RQ | Reportable Quantity (RQ) |
| --- | --- | --- | --- |
| Sodium Dodecylbenzene Sulfonate 25155-30-0 | 1000 lb | - | RQ 1000 lb final RQ RQ 454 kg final RQ |

# US State Regulations

## California Proposition 65

WARNING: This product can expose you to chemicals including Acetic acid, 2,2-dichloro. which is known to the state of California to cause cancer, or birth defects or other reproductive harm. For More Information go to www.P65Warnings.ca.gov.

## U.S. State Right-to-Know Regulations

| Chemical Name | New Jersey | Massachusetts | Pennsylvania |
| --- | --- | --- | --- |
| Sodium Dodecylbenzene Sulfonate 25155-30-0 | X | X | X |
| Sodium Sulfate 7757-82-6 | - | X | X |

U.S. EPA Label Information

EPA Pesticide Registration Number Not Applicable

16. OTHER INFORMATION

| NFPA | Health hazards 3 | Flammability 0 | Instability 0 | Physical and Chemical |
| --- | --- | --- | --- | --- |
|  |  |  |  | Properties - |
| HMIS | Health hazards 3 | Flammability 0 | Physical hazards 0 | Personal protection B |

Issue Date	21-Mar-2018

Revision Date	21-Mar-2018

## Revision Note

No Information available

## Disclaimer

The information provided in this Safety Data Sheet is correct to the best of our knowledge, information and belief at the date of its publication. The information given is designed only as a guidance for safe handling, use, processing, storage, transportation, disposal and release and is not to be considered a warranty or quality specification. The information relates only to the specific material designated and may not be valid for such material used in combination with any other materials or in any process, unless specified in the text.

End of Safety Data Sheet