# P&C SUPPLIES

# Building 35 Unit 3

# Tonique Darling Hwy

# Nassau, Bahamas

# 242-364-3505

Nassau, Bahamas

MATERIAL SAFETY DATA SHEET

Revision Date: 10/15/2014	MSDS No. N091

#### Emergency Phones: (242) 364-3603

PLEASE NOTE: This MSDS is being provided to your company for the purpose of providing current health and safety information to your management and for your employees who work with this material. Please read the information on these sheets, and then provide this information to those people at your company whose responsibility it is to comply with FEDERAL and STATE RIGHT-TO-KNOW regulations. Also make this information available to any employee who requests it. It is your obligation to comply with these regulations.

## SECTION I - PRODUCT IDENTITY

### PRODUCT NAME: White Lightning Tile & Grout Whitener

#### Formula: Mixture sodium hypochlorite, caustic soda and detergent in water Chemical Type: As above

HMIS RATINGS

| Health = 2 (Moderate) | Flammability = 0 (Minimal) |
| --- | --- |
| Reactivity = 0 (Minimal) | Protection = B (Goggles & Gloves) |

SECTION II - HAZARDOUS INGREDIENTS

##### CARCINOGEN

Sodium hypochlorite CAS No. 7681-52-9

Sodium hydroxide CAS No. 1310-73-2

##### PERCENT

1-10%

1-5%

TLV

None

2mg/m3 ceiling

##### (OSHA,TP,IARC)

No No

Appearance: Clear Liquid Odor: Rain Fresh

pH: > 12.0

Water Solubility: Complete

Viscosity @ 25oC: < 25 cps (Brookfield)

### SECTION III - CHEMICAL AND PHYSICAL

Boiling Point: Est. 212oF. Melting Point: N/A

Spec. Gravity (H20 = 1): 1.086 Vapor Pressure (Mm Hg): N/A Voc Content: None

### SECTION IV - FIRE AND EXPLOSION HAZARDS

Flash Point (Method):

N/A

Explosion Limits:

Upper: N/A Lower: N/A

Extinguishing Media: Product is non-flammable

Special Firefighting Procedures and Hazards: Minimize prolonged skin and eye contact. Wear eye protection if exposure is likely. Heated product may release chlorine.

Stable:

Unstable:

SECTION V - REACTIVITY INFORMATION

##### Precautions:

Incompatibility: Strong acids, materials not resistant to alkalis, ammonia compounds. Hazardous Decomposition Products: When heated, chlorine.

Hazardous Polymerization: Occurs:

Does Not Occur:

### SECTION VI - HEALTH HAZARDS - PROTECTIVE MEASURES - FIRST AID

Inhalation:

Breathing of mists may cause irritation. Wear approved alkali mist respirator if necessary. Give artificial respiration or oxygen if needed. If needed get prompt medical attention.

Skin:

Extended contact may cause irritation or defatting. Wear protective gloves if necessary. Flush skin with water for 15 minutes. Get medical attention for any irritation.

Eyes:

Can cause irritation. Wear goggles as needed. Flush immediately with water for 15 minutes. Get prompt medical attention if needed.

Ingestion:

May cause irritation and possible burning in mouth and throat and a mild gastric disturbance. Drink lots of water. Do not induce vomiting. Get medical attention if effects persist.

Most likely routes of entry: Skin, Eyes.

Other Important Medical or Precautionary Information: Provide convenient eyewash station. Do no mix this material with any other chemicals as a possibility of a toxic gas could be formed.

### SECTION VII - PRECAUTIONS FOR SAFE HANDLING AND USE

Spills and Leaks: Small spills can be flushed into normal drainage or into ground with copious amounts of water, or taken up with absorbent material. Larger spills should be contained by diking or other methods and held for disposal or reuse.

Storage and Handling: Check daily for any leaks from containers, vessels, pumps, and piping.

Waste Disposal: Product may be disposable in sewers if local regulations permit. Otherwise send to a licensed treatment facility. The material is toxic to fish.

Empty Containers: Rinse well before handling and disposal. Observe label precautions.

Other Precautions: None

### SECTION VIII - REGULATORY INFORMATION

Reportable for SARA Title III, S.313 (Form R): None

The information herein has been compiled from sources believed to be reliable and is accurate to the best of our knowledge. However, P&C Janitorial Service & Supply’s cannot give any guarantees regarding information from other sources, and expressly does not make any warranties, nor assumes any liability, for its use.